| Command | HEX Code | Notes |
| --- | --- | --- |
| VOLUME CONTROL |  |  |
| Volume Up | 0x 807F 03FC; Repeat Command Issued when button is held | Volume control responds on first press, then begins to ramp up after being held for >0.5 seconds. |
| Volume Down | 0x 807F 06F9; Repeat Command Issued when button is held | Volume control responds on first press, then begins to ramp up after being held for >0.5 seconds. |
| Mute/Unmute (toggle) | 0x 807F 01FE |  |
| Discreet Mute | 0x 807F 1BE4 | Installer Code |
| Discreet Unmute | 0x 807F 1DE2 | Installer Code |
| POWER CONTROL |  |  |
| Power On/Standby (Toggle) | 0x 807F 1AE5 |  |
| Discreet Standby (Discreet “off”) | 0x 807F 1EE1 | Installer Code |
| Discreet On (Discreet “on”) | 0x 807F 1CE3 | Installer Code |
| SOURCE SELECTION |  |  |
| Source (scroll) | 0x 807F 02FD |  |
| On/Optical* (Digital) | 0x 807F 0CF3 | If in STANDBY, system wakes and selects desired source. |
| On/Bluetooth* | 0x 807F 0DF2; Repeat Command Issued when button is held | If in STANDBY, system wakes and selects desired source. |
|  |  | When held for 3 seconds, system enters Bluetooth Pairing mode, |
|  |  | (i.e. Bluetooth is discoverable) |
| On/Aux* (RCA) | 0x 807F 0EF1 | If in STANDBY, system wakes and selects desired source. |
| On/Aux* (Phono) | 0x 807F 0FF0 | If in STANDBY, system wakes and selects desired source. |
| On/Aux* (3.5mm) | 0x 807F 0EF1 | If in STANDBY, system wakes and selects desired source. |
| On/COAX* (Digital) | 0x 807F 11EE | If in STANDBY, system wakes and selects desired source. |
| On/USB* | 0x 807F 12ED | If in STANDBY, system wakes and selects desired source. |
| On/HDMI 1* | 0x 807F 15EA | If in STANDBY, system wakes and selects desired source. |
| On/HDMI 2* | 0x 807F 16E9 | If in STANDBY, system wakes and selects desired source. |
| On/HDMI 3* | 0x 807F 17E8 | If in STANDBY, system wakes and selects desired source. |
| On/HDMI 4* | 0x 807F 18E7 | If in STANDBY, system wakes and selects desired source. |
| On/HDMI 5 (ARC/TV)* | 0x 807F 19E6 | If in STANDBY, system wakes and selects desired source. |
| SOURCE SCROLL DOWN | 0x 807F 6B94 |  |
| SET DISCOVERABLE MODE | 0x 807F 6C93 |  |
| * System wakes up and selects the desired source; i.e. no discreet "on" command needed. |  |  |
| MEDIA CONTROLS & NAVIGATION |  |  |
| Play/Pause (toggle) | 0x 807F 21DE |  |
| Next Track | 0x 807F 22DD; Repeat Command Issued when button is held | Product optionally uses holding Next Track to do Fast FWD |
| Prev Track | 0x 807F 23DC; Repeat Command Issued when button is held | Product optionally uses holding Next Track to do Fast REV |
| Fast FWD | 0x 807F 24DB; Repeat Command Issued when button is held |  |
| Fast REV | 0x 807F 25DA; Repeat Command Issued when button is held |  |
| Repeat (scroll/toggle) | 0x 807F 26D9 |  |
| Repeat 1 | 0x 807F 27D8 |  |
| Repeat All | 0x 807F 28D7 |  |
| A‐B | 0x 807F 29D6 |  |
| Reserved for other play control | 0x 807F 2AD5 |  |
| Stop | 0x 807F 20DF |  |
| DPAD UP | 0x 807F 609F; Repeat Command Issued when button is held |  |
| DPAD DOWN | 0x 807F 619E; Repeat Command Issued when button is held |  |
| DPAD LEFT | 0x 807F 629D; Repeat Command Issued when button is held |  |
| DPAD RIGHT | 0x 807F 639C; Repeat Command Issued when button is held |  |
| DPAD OK | 0x 807F 649B |  |
| MENU START | 0x 807F 659A |  |
| MENU EXIT | 0x 807F 6699 |  |
| MENU RETURN | 0x 807F 6798 |  |
| Number Key 0 | 0x 807F 30CF |  |
| Number Key 1 | 0x 807F 31CE |  |
| Number Key 2 | 0x 807F 32CD |  |
| Number Key 3 | 0x 807F 33CC |  |
| Number Key 4 | 0x 807F 34CB |  |
| Number Key 5 | 0x 807F 35CA |  |
| Number Key 6 | 0x 807F 36C9 |  |
| Number Key 7 | 0x 807F 37C8 |  |
| Number Key 8 | 0x 807F 38C7 |  |
| Number Key 9 | 0x 807F 39C6 |  |
| EQ & DSP SETTINGS |  |  |
| EQ1-Dialog (toggle) | 0x 807F 2BD4 |  |
| EQ2-Night (toggle) | 0x 807F 2CD3 |  |
| 3D/Surround Mode (scroll/toggle) | 0x 807F 04FB; Repeat Command Issued when button is held | When held for 3 seconds, system enters surround level adjust mode. |
| EQ3 | 0x 807F 2DD2 |  |
| EQ4 | 0x 807F 2ED1 |  |
| EQ5 | 0x 807F 2FD0 |  |
| Treble Up | 0x 807F 08F7; Repeat Command Issued when button is held |  |
| Treble Down | 0x 807F 0BF4; Repeat Command Issued when button is held |  |
| Midrange Up | 0x 807F 05FA; Repeat Command Issued when button is held |  |
| Midrange Down | 0x 807F 09F6; Repeat Command Issued when button is held |  |
| Bass Up | 0x 807F 07F8; Repeat Command Issued when button is held | Same as Wireless Sub Up |
| Bass Down | 0x 807F 0AF5; Repeat Command Issued when button is held | Same as Wireless Sub Down |
| EQ INCREMENT (Scroll) | 0x 807F 3AC5 |  |
| EQ Off/Reset | 0x 807F 3BC4 |  |
| SURR MODE = STEREO | 0x 807F 40BF |  |
| SURR MODE = DIRECT | 0x 807F 41BE |  |
| SURR MODE = ALL CH STEREO | 0x 807F 42BD |  |
| SURR MODE = PLII MUSIC | 0x 807F 43BC |  |
| SURR MODE = PLII MOVIE | 0x 807F 44BB |  |
| SURR MODE = PLII GAME | 0x 807F 45BA |  |
| SURR MODE = NEO6 CINEMA | 0x 807F 46B9 |  |
| SURR MODE = NEO6 MUSIC | 0x 807F 47B8 |  |
| SURR MODE ‐ RESERVED | 0x 807F 48B7 |  |
| SURR MODE ‐ RESERVED | 0x 807F 49B6 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4AB5 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4BB4 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4CB3 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4DB2 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4EB1 |  |
| SURR MODE ‐ RESERVED | 0x 807F 4FB0 |  |
| DOLBY NIGHT MODE TOGGLE | 0x 807F 5DA2 |  |
| DOLBY NIGHT MODE ON | 0x 807F 5EA1 |  |
| DOLBY NIGHT MODE OFF | 0x 807F 5FA0 |  |
| SUBWOOFER CONTROL |  |  |
| Bass/Wireless Sub Up | 0x 807F 07F8; Repeat Command Issued when button is held | For Wireless Subwoofers (Soundbars/WiSA) |
| Bass/Wireless Sub Dn | 0x 807F 0AF5; Repeat Command Issued when button is held | For Wireless Subwoofers (Soundbars/WiSA) |
| Bass/Wired Sub Up | 0x 807F 3CC3; Repeat Command Issued when button is held | For Subwoofer Line Out (RCA) |
| Bass/Wired Sub Dn | 0x 807F 3DC2; Repeat Command Issued when button is held | For Subwoofer Line Out (RCA) |
| Sub Reset | 0x 807F 3EC1 |  |
| Start Sub Level Control Mode | 0x 807F 3FC0 |  |
| LED & DISPLAY CONTROL |  |  |
| Display/LED Brightness On/Dim/Off (scroll/toggle) | 0x 807F 1FE0 |  |
| LED MODE = 100% | 0x 807F 6897 |  |
| LED MODE = 50% | 0x 807F 6996 |  |
| LED MODE = OFF | 0x 807F 6A95 |  |
| UNIVERSAL REMOTE CODES |  |  |
| Volume Up | 0x 856A 9F60; 0x A55A 0AF5; 0x 859A 817E | Product must respond correctly to all of these IR Codes |
| Volume Down | 0x 856A 9E61; 0x A55A 0BF4; 0x 859A 827D | Product must respond correctly to all of these IR Codes |
| Mute | 0x 856A 9966; 0x A55A 12ED; 0x 859A 8778 | Product must respond correctly to all of these IR Codes |
|  |  | Product must ignore Mute repeat commands |
| Volume Up | 0x 02A1 DF3B; 0x 0B4F 24DB | Product optionally responds to these IR Codes |
| Volume Down | 0x 02A1 DF3F; 0x 0B4F 23DC | Product optionally responds to these IR Codes |
| Mute | 0x 02A1 DF5F; 0x 0B4F 03FC | Product optionally responds to these IR Codes |
|  |  | Product must ignore Mute repeat commands |
| ALTERNATE CODE SET |  |  |
| Change Code Set |  | Alternate code set ON Hold TV and HDMI2 buttons for 3 seconds to engage. |
| Change Code Set |  | Alternate code set OFF Hold TV and HDMI2 buttons for 3 seconds to turn off |